# Laser Beam

USE MANUAL

Catalogue

Safety guide............................................................................3

Specification parameters.........................................................4

Insall........................................................................................6

Power connection....................................................................7

Signal connection....................................................................9

Lighting setting........................................................................10

Channel data...........................................................................15

Control line diagram................................................................18

Maintenance guide..................................................................19

Thank you very much for purchasing this product. Before you use our product, please carefully read carefully and fully understand all the safety instructions

recorded in this book to avoid the potential dangers that may cause bodily injury,

property damage or damage to the product. The Company shall not be liable for any direct losses and associated losses caused by the failure to operate as instructed in  the instructions.

WARNING

SAFETY GUIDE

Please read the instruction manual carefully, which contains important information about installation, operation, maintenance, etc

This product is intended for professional use only and is prohibited for family use

Please keep this manual for future reference, if you sell the machine to another user, please ensure that the user also gets this manual.

u Open the package before use and carefully check if the machine is damaged during transportation.

u Non-professional operators are not allowed to install and operate the machine.

u Disconnect the external power supply when opening the housing or replacing the parts, or when the lamp is not in use.

u Ensure that the equipment is reliable and use the AC power supply matching the lamp voltage frequency with overload and grounding protection.

u Before use, ensure that the cable is in normal condition and suitable for the flow of the connected equipment.

u Avoid having the power cord going around with the surrounding cables.

u Ensure that the equipment is reliably grounded.

u If the power cable or plug is damaged, the lamp is wet or the power supply is obviously overheated, please disconnect the power supply immediately.

u Avoid any flammable liquid, water, or metal material from entering the machine. Once it happens, please cut off the power supply immediately.

u Do not look directly into the light port and the illuminated light source.

u Do not use damaged housing, light shield, lens, protective screen, or heat lamps.

u If the protective screen, lens or ultraviolet screen on the lamp produces visible damage, that is, the damage to the degree of failure, such as cracks or deep marks, should be replaced in time.

u Do not use when the ambient temperature exceeds 40℃ .

u When the external temperature of the lamps during the use is 65℃, to avoid contact with the human body or other objects. Please cool the lamps fully before handling.

u Ensure that combustible materials (such as fabric, wood, paper, etc.) away from the lamps at least 0. 5 meters, flammable items as far as possible away from the equipment, ensure that the inlet and outlet at least 0. 5 meters of space

u Do not directly illuminate surfaces within 10 meters.

u During the day, cover the lens or turn the lens direction to prevent direct sunlight from reaching the lens, as it can focus on the sun and create a potential fire hazard.

u In any case, do not modify the lamp, do not install non-original parts, do not paste any filter paper on the lens or other optical accessories.

u Disconnect the power supply before maintaining the clean lamps.

u Do not attempt to short circuit the fuse or the temperature control switch.

u Do not carry the lamps and lanterns alone when working at a height.

u Use two evenly distributed lamp hooks on the hanger, and do not use a single lamp hook.

u Ensure that the support and hanging system can support more than 10 times the weight of the lamp.

u When used, a safety rope must be installed, which must be able to withstand 10 times the weight of the lamp.

u Ensure that the external fittings and hangings are fully fastened.

u For installation, maintenance and moving lamps, stable platform shall be provided with safety fence.

u For related operations not described in this article, please consult the Authorized after-sales department.

# Specification parameters

Voltage: AC100-240V  50 / 60Hz

Maximum power: 400W

Light source: a 300W white laser light source

Light source life: 12,000 hours

Control: 14 / 16 channel, DMX512, RDM.

Color: one color plate, 14 colors + white light, half-color and two-way color rainbow effect

Pattern: a fixed pattern plate, 19 kinds of patterns + white light, running water, jitter effect

Prism: 8 prism + 24 prism: can be two-way rotation, can switch independently, can stack

Optics: high-precision glass optical lens, optical diameter of 165mm, electric focus

Effect: nebulization effect

Movement: horizontal 540 degrees, 8 / 16 Bit resolution, 360 degrees infinite rotation 252 degrees vertical, 8 / 16 Bit resolution

Vertical magnetic encoder; three-phase stepper motor

Display: 1.7-inch display screen, touch button, Chinese and English interface.

Software upgrade: Software can be updated through a DMX connection

Waterproof power supply connector input / output

Three-core / five-core waterproof Canon head socket Fold hook is free

Protection level: IP66

Dimensions: 348 * 250 * 624mm

Net weight: 21.6Kg

# INSTALL

| Δ | WARMING: When hanging lamps, additional safety rope must be installed to prevent the failure of lamp hook, |
| --- | --- |
| The safety rope must be attached to the mounting position of the safety rope, and must not be installed on the handling handle When the lamp is not placed steadily, the movement of the lamp head may drive the movement of the base, Therefore, do not start the lamp without confirming the stable placement or hanging of the lamp. Do not hang the side installation and use of lamps! Cover the lens or turn the direction to avoid direct sunlight into the lens, Because the camera can focus on sunlight as a potential fire danger. |  |

Installation considerations:

1． Confirm that the hanging lamp hook and accessories are not damaged and can withstand 10 times the total weight of the lamps, cables and accessories,

2． For installation, maintenance and moving lamps, stable platform shall be provided with safety fence

3． Tie the safety rope through the hole in the lamp base to the lamp holder, not to the handle

4． Check that there is no combustible material within 0. 5 meters of the lamp shell, do not be blocked within 10 meters of the lamp column directly, and no flammable substances nearby

5． Check that the lights are not possible to collide with each other during movement

6． Check that the other lamp has not directly illuminate on the lamp, the strong light will damage the protective shell of the lamp

7． Make sure that the installation surface is horizontal, so as to avoid sliding when the lamp is running

warn: To prevent electric shock, the lamp must be grounded, and the main power supply must be safe and disconnected Switch and ground protection Please connect the lamp directly to AC power supply, not the silicon box to avoid damage lamps and lanterns When the power supply of the lamps is connected in series, the maximum current of the power supply seat shall not be greater than 1 6 A

Power connection

A plug matching with the lamp power interface has been installed on one end of the power cord, and a suitable power plug with grounding shall be installed on the other end. Installation must follow the manufacturers instructions. The table below gives some tips for plug and pin connections. If not sure or questionable, consult a qualified electrician.

AC100V The power cord can connect up to 4 lamps in series

AC220V The power cord can connect up to 8 lamps in series

Signal connection

DMX signal via XLR Cannon insert, defined as follows:

1. Connect a DMX signal line from the controller to the lamp signal input port

2. Connect one DMX signal line to the next lamp from the lamp signal output port

3. The DMX signal line can only be connected in series. In parallel, please use the signal distributor

3. Interspan a terminal resistance between the 2 and 3 feet of the last lamp signal output

Reliability tips

l Using a shielded twisted pair designed for RS-485, the connection distance of the  signal   is  generally   150  meters,  and  a  signal  amplifier   is  used  for long-distance transmission

l Branch driver (signal in parallel) requires a signal distributor

l A 120 ohm 1 / 4 watt resistance is crossed between the 2 and 3 feet of the last  lamp signal output to avoid signal  reflection  interference, so that the resistance should be connected to the respective terminals

Lighting setting

l 1- -MENU (menu key) 2- -UP (upper key) 3- -DOWN (lower key) 4- -ENTER (confirmation key)

l Key lock: touch the menu button and confirmation button in 3 seconds

Menu structure

| DMX地址 DMX Addr | 红色背景时地址码加减1，黑色背景时地址码按通道模式加减 |  |  |  |
| --- | --- | --- | --- | --- |
| 通道模式 Channel Mode | 16 |  |  |  |
|  | 14 |  |  |  |
| 运行模式 Run Mode | DMX |  |  |  |
|  | 用户 User |  |  |  |
|  | 自走 Auto |  |  |  |
| 手控 Manual | 锁定手控 Lock Manual Ctrl | 否 No |  |  |
|  |  | 是 Yes |  |  |
|  | X轴 Pan |  |  |  |
|  | Y轴 Tilt |  |  |  |
|  | X轴旋转 Pan Rot |  |  |  |
|  | 颜色 Color |  |  |  |
|  | 频闪 Strobe |  |  |  |
|  | 调光 Dimmer |  |  |  |
|  | 固图 Gobo |  |  |  |
|  | 棱镜 Prism |  |  |  |
|  | 棱镜自转 Prism Rot |  |  |  |
|  | 雾化 Frost |  |  |  |
|  | 调焦 Focus |  |  |  |
| 复位 Reset | 整机复位 Reset All |  |  |  |
|  | XY轴 Pan Tilt |  |  |  |
|  | 头部复位 Head Reset |  |  |  |
| 运行设置 Run Settings | 主从模式 M/S Mode | 从机 Slave |  |  |
|  |  | 主机 Master |  |  |
|  | XY轴设置 Pan/Tilt Settings | X轴反向 Pan Reverse | 否 No |  |
|  |  |  | 是 Yes |  |
|  |  | Y轴反向 Tilt Reverse | 否 No |  |
|  |  |  | 是 Yes |  |
|  | 调光曲线 Dim Curve | 线性 Linear |  |  |
|  |  | 对数 Log |  |  |
|  |  | S曲线 S-Curve |  |  |
|  |  | 开方 Square Root |  |  |
|  |  | 平方 Square |  |  |
|  | LED模式 LED Mode | 标准 Standard |  |  |
|  |  | 高亮 High Light |  |  |
|  |  | 静音 Silent |  |  |
|  | 自编程 User Program | 手工编辑 Manual | 步数01 Step01 . . . . . . 步数12 Step12 | 时间 Time |
|  |  |  |  | 通道01 Chn01 |
|  |  |  |  | 通道02 Chn02 |
|  |  |  |  | 通道03 Chn03 |
|  |  |  |  | 通道04 Chn04 |
|  |  |  |  | 通道05 Chn05 |
|  |  |  |  | 通道06 Chn06 |
|  |  |  |  | 通道07 Chn07 |
|  |  |  |  | 通道08 Chn08 |
|  |  |  |  | 通道09 Chn09 |
|  |  |  |  | 通道10 Chn10 |
|  |  |  |  | 通道11 Chn11 |
|  |  |  |  | 通道12 Chn12 |
|  |  |  |  | 通道13 Chn13 |
|  |  |  |  | 通道14 Chn14 |
|  |  | 录制DMX Record | 选择步数 Select Step | 01--012 |
|  |  |  | 时间 Time |  |
|  |  |  | 录制 Record |  |
|  |  | 全部清零 Clear | 密码 Input PSW |  |
|  | 无信号时 If No DMX | 开机时 Power On | 居中 Middle |  |
|  |  |  | 无效果 No Effect |  |
|  |  |  | 用户 User |  |
|  |  |  | 自走 Auto |  |
|  |  | 运行时 Run Time | 保持 Keep |  |
|  |  |  | 闭关 Shutter Off |  |
| 高级设置 Advanced Settings | 偏移量 Adj Offset | 密码 Input PSW | 000 |  |
|  | 显示屏 LCD Display | 休眠 Sleep | 30秒 30Sec |  |
|  |  |  | 2分钟 2min |  |
|  |  |  | 10分钟 10min |  |
|  |  |  | 无 No |  |
|  |  | 翻转 Flip | 否 No |  |
|  |  |  | 是 Yes |  |
|  |  | 自动锁屏 Auto Lock | 是 Yes |  |
|  |  |  | 否 No |  |
|  | 忽略错误 Ignore Err | 码盘波形 Raster Waveform | 否 No |  |
|  |  |  | 是 Yes |  |
|  | 工厂设置 Factory Setting | 密码 Input PSW |  |  |
|  | 微调恢复 Adj Recover | 密码 Input PSW |  |  |
|  | 恢复出厂设置 Restore Defaults | 密码 Input PSW | 221 |  |
| 设备信息 Device Info | 光源使用时间 Lamp Hours |  |  |  |
|  | 设备使用时间 Fixture Hours |  |  |  |
|  | 光源时间清零 Clear Lamp Hours | 密码 Input PSW |  |  |
|  | 头部风机01 Head Fan1 |  |  |  |
|  | 头部风机02 Head Fan2 |  |  |  |
|  | 头部风机03 Head Fan3 |  |  |  |
|  | 激光温度 Laser Temp |  |  |  |
|  | 色轮温度 Color Wheel Temp |  |  |  |
|  | DMX值 DMX Value |  |  |  |
|  | 错误 Error |  |  |  |
|  | 测试模式 Test Mode | 加热棒 Heating Rod | 关 Off |  |
|  |  |  | 开 On |  |
|  |  | 风机测试 Fan Test | 风机01 Fan01 | 0-101% |
|  |  |  | 风机02 Fan02 | 0-101% |
|  |  |  | 风机03 Fan03 | 0-101% |
|  |  |  | 风机04 Fan04 | 0-101% |
|  |  |  | 风机05 Fan05 | 0-101% |
|  |  |  | 风机06 Fan06 | 0-101% |
| 语言（Lang） | 中文 |  |  |  |
|  | English |  |  |  |

Channel data

| 14CH | 16CH | FUNCTION | VALUE |  |
| --- | --- | --- | --- | --- |
| 1 | 1 | 颜色 Color | 0--4 | WHITE |
|  |  |  | 5--8 | WHITE+RED |
|  |  |  | 9--12 | RED |
|  |  |  | 13--17 | RED+ORANGE |
|  |  |  | 18--21 | ORANGE |
|  |  |  | 22--25 | ORANGE+AQUAMARINE |
|  |  |  | 26--29 | AQUAMARINE |
|  |  |  | 30--34 | AQUAMARINE+GREEN |
|  |  |  | 35--38 | GREEN |
|  |  |  | 39--42 | GREEN+LIGHT GREEN |
|  |  |  | 43--46 | LIGHT GREEN |
|  |  |  | 47--51 | LIGHT GREEN+LAVENDER |
|  |  |  | 52--55 | LAVENDER |
|  |  |  | 56--59 | LAVENDER+PINK |
|  |  |  | 60--63 | PINK |
|  |  |  | 64--68 | PINK+YELLOW |
|  |  |  | 69--72 | YELLOW |
|  |  |  | 73--76 | YELLOW+MAGENTA |
|  |  |  | 77--81 | MAGENTA |
|  |  |  | 82--85 | MAGENTA+CYAN |
|  |  |  | 86--89 | CYAN |
|  |  |  | 90--93 | CYAN+CTO 260 |
|  |  |  | 94--98 | CTO 260 |
|  |  |  | 99--102 | CTO 260+CTO 190 |
|  |  |  | 103--106 | CTO 190 |
|  |  |  | 107--110 | CTO 190+CTB 8000 |
|  |  |  | 111--115 | CTB 8000 |
|  |  |  | 116--119 | CTB 8000+BLUE |
|  |  |  | 120--123 | BLUE |
|  |  |  | 124--127 | BLUE+WHITE |
|  |  |  | 128--255 | ROTATION SLOW--FAST |
| 2 | 2 | 频闪 Strobe | 0--3 | CLOSED |
|  |  |  | 4--103 | STROBE SLOW--FAST |
|  |  |  | 104--107 | OPEN |
|  |  |  | 108--207 | PULSATION SLOW--FAST |
|  |  |  | 208--212 | OPEN |
|  |  |  | 213--251 | RANDOM STROBE |
|  |  |  | 252--255 | OPEN |
| 3 | 3 | 调光 Dimmer | 0--255 | DIMMER 0--100% |
| 4 | 4 | 图案 | 0--3 | OPEN |

|  |  | Gobo | 4--7 | GOBO1 |
| --- | --- | --- | --- | --- |
|  |  |  | 8--11 | GOBO2 |
|  |  |  | 12--15 | GOBO3 |
|  |  |  | 16--19 | GOBO4 |
|  |  |  | 20--23 | GOBO5 |
|  |  |  | 24--27 | GOBO6 |
|  |  |  | 28--31 | GOBO7 |
|  |  |  | 32--35 | GOBO8 |
|  |  |  | 36--39 | GOBO9 |
|  |  |  | 40--43 | GOBO10 |
|  |  |  | 44--47 | GOBO11 |
|  |  |  | 48--51 | GOBO12 |
|  |  |  | 52--55 | GOBO13 |
|  |  |  | 56--59 | GOBO14 |
|  |  |  | 60--63 | GOBO15 |
|  |  |  | 64--67 | GOBO16 |
|  |  |  | 68--71 | GOBO17 |
|  |  |  | 72--75 | GOBO18 |
|  |  |  | 76--79 | GOBO19 |
|  |  |  | 80--117 | ROTATION FAST--SLOW |
|  |  |  | 118--121 | STOP |
|  |  |  | 122--159 | ROTATION SLOW--FAST |
|  |  |  | 160--165 | GOBO2 SHAKE |
|  |  |  | 166--171 | GOBO3 SHAKE |
|  |  |  | 172--177 | GOBO4 SHAKE |
|  |  |  | 178--183 | GOBO5 SHAKE |
|  |  |  | 184--189 | GOBO6 SHAKE |
|  |  |  | 190--195 | GOBO7 SHAKE |
|  |  |  | 196--201 | GOBO9 SHAKE |
|  |  |  | 202--207 | GOBO10 SHAKE |
|  |  |  | 208--213 | GOBO11 SHAKE |
|  |  |  | 214--219 | GOBO12 SHAKE |
|  |  |  | 220--225 | GOBO13 SHAKE |
|  |  |  | 226--231 | GOBO15 SHAKE |
|  |  |  | 232--237 | GOBO16 SHAKE |
|  |  |  | 238--243 | GOBO17 SHAKE |
|  |  |  | 244--249 | GOBO18 SHAKE |
|  |  |  | 250--255 | GOBO19 SHAKE |
| 5 | 5 | 棱镜 Prism | 0--63 | OPEN |
|  |  |  | 64--127 | PRISM1 |
|  |  |  | 128--191 | PRISM2 |
|  |  |  | 192--255 | PRISM1+PRISM2 |

| 6 | 6 | 棱镜旋转 Prism Rot | 0--63 | POSITION INDEX |
| --- | --- | --- | --- | --- |
|  |  |  | 64--127 | CCW FAST--SLOW |
|  |  |  | 128--191 | CW SLOW--FAST |
|  |  |  | 192--207 | ROTATE BACK AND FORTH 90 °,SLOW--FAST |
|  |  |  | 208--223 | ROTATE BACK AND FORTH 180 °,SLOW--FAST |
|  |  |  | 224--239 | ROTATE BACK AND FORTH 270 °,SLOW--FAST |
|  |  |  | 240--255 | ROTATE BACK AND FORTH 360 °,SLOW--FAST |
|  | 7 | 棱镜宏 Prism Macro | 0--15 | UNUSED RANGE |
|  |  |  | 16--55 | PRISM 1 FREE SWITCHING,SLOW--FAST |
|  |  |  | 56--95 | PRISM 2 FREE SWITCHING,SLOW--FAST |
|  |  |  | 96--135 | PRISM 1 AND 2 FREE SWITCHING,SLOW--FAST |
|  |  |  | 136--175 | PRISM 1 SELECTION，PRISM 2 FREE SWITCHING,SLOW--FAST |
|  |  |  | 176--215 | PRISM 2 SELECTION，PRISM 1 FREE SWITCHING,SLOW--FAST |
|  |  |  | 216--255 | PRISM 1 AND PRISM 2 SWITCH TO EACH OTHER,SLOW--FAST |
| 7 | 8 | 雾化 Frost | 0--127 | OPEN |
|  |  |  | 128--255 | FROST INSERTED |
| 8 | 9 | 调焦 Focus | 0--255 | FAR--NEAR |
| 9 | 10 | 水平 Pan | 0--255 | PAN MOVEMENT |
| 10 | 11 | 水平微调 Pan Fine | 0--255 | PAN MOVEMENT FINE |
| 11 | 12 | 垂直 Tilt | 0--255 | TILT MOVEMENT |
| 12 | 13 | 垂直微调 Tilt Fine | 0--255 | TILT MOVEMENT FINE |
| 13 | 14 | 水平旋转 Pan Rot | 0--127 | UNUSED RANGE |
|  |  |  | 128--189 | CCW FAST--SLOW |
|  |  |  | 190--193 | STOP |
|  |  |  | 194--255 | CW SLOW--FAST |
| 14 | 15 | 复位 Reset | 0--25 | UNUSED RANGE |
|  |  |  | 26--76 | EFFECT RESET |
|  |  |  | 77--127 | P/T RESET |
|  |  |  | 128--255 | RESET ALL |
|  | 16 | 运行模式 Run Mode | 0--50 | UNUSED RANGE |
|  |  |  | 51--100 | Silent，Stop for 3 seconds and enable |
|  |  |  | 101--150 | Standard，Stop for 3 seconds and enable |
|  |  |  | 151--200 | High Light，Stop for 3 seconds and enable |
|  |  |  | 201--255 | UNUSED RANGE |

# Control line diagram

MAINTIAN

Lamps require daily cleaning. The time depends on the following factors related to the use environment and the cleaning cycle:

l Use a smoke machine, a fog machine

l High wind speed (e. g. inlet and outlet)

l Dust (e. g. stage effects, the structure of the building, and occasions required to simulate the natural outdoor environment)

l If one or more factors exist, check whether the lamp is cleaned after 100 hours of operation and check it periodically

All repairs not mentioned here shall be consulted by the technician.

|  | WARN: Disconnect the AC power supply before opening the lamp |
| --- | --- |
| housing and cool the lamp completely |  |

l Be careful when cleaning the optical components, and work in a clean and bright place. Optical coated surfaces are fragile and easy to wipe flowers. Do not use solvents that can damage the plastic or colored surfaces.

l To maintain proper cooling, the fans and vents must be removed periodically

l Clean optical parts

l 1. Cut off the power supply and cool the lamp completely. Open the lamp enclosure.

l 2. Spray the air compressor.

l 3. Touch cotton paper with glass cleaner or pure water on cotton paper to wipe away stubborn stains. But can not wipe the surface, should gently repeat wipe.

l 4. Remove lampblack debris with alcohol cotton cloth. Glass cleaners can also be used,  but  use  pure  water to clean  up  the  residue.  Slowly  clean from  inside  to outside.

l 5. Blow it dry with a clean and soft cloth or an air compressor.

l Clean the fan and air vents

l Use soft brushes, cotton cloth, vacuum cleaner or air compressor to remove dust from fans and vents.