400LED BSW CMY WITH LINEAR CTO

# GENERAL INFORMATION

INTRODUCTION

Please read and understand the instructions in this manual carefully and thoroughly before attempting

to operate this device. These instructions contain important safety and use information.

This product is intended for use by professionally trained personnel only, and is not suitable for

private use.

Unpacking

Every device has been thoroughly tested and has been shipped in perfect operating condition. Carefully

check the shipping carton for damage that may have occurred during shipping. If the carton is damaged,

carefully inspect the device for damage, and be sure all accessories necessary to install and operate

the device have arrived intact. In the event damage has been found or parts are missing, please contact

our customer support team for further instructions. Please do not return this device to your dealer

without first contacting customer support. Please do not discard the shipping carton in the trash. Please

recycle whenever possible.

IMPORTANT NOTICE!

THERE ARE NO USER SERVICEABLE PARTS INSIDE THIS UNIT.

DO NOT ATTEMPT ANY REPAIRS YOURSELF; DOING SO WILL VOID YOUR

MANUFACTURER'S

WARRANTY. DAMAGES RESULTING FROM MODIFICATIONS TO THIS FIXTURE AND/OR THE DISREGARD OF SAFETY INSTRUCTIONS AND GUIDELINES IN THIS MANUAL VOID THE

MANUFACTURER’S WARRANTY AND ARE NOT SUBJECT TO WARRANTY CLAIMS AND/OR REPAIRS.

# SAFETY  GUIDELINES

This device is a sophisticated piece of electronic equipment. To guarantee a smooth operation, it is important to follow all instructions and guidelines in this manual. ADJ PRODUCTS, LLC. is not responsible for injury and/or damages resulting from the misuse of this device due to the disregard of the information printed in this manual. Only the original included parts and/or accessories for this device should be used. Any modifications to the device, included and/or accessories will void the original manufactures warranty and increase the risk of damage and/or personal injury.

|  | PROTECTION CLASS 1 - FIXTURE MUST BE PROPERLY GROUNDED THERE ARE NO USER SERVICEABLE PARTS INSIDE THIS UNIT. |
| --- | --- |
| Δ | DONOTATTEMPTANYREPAIRSYOURSELF;DOINGSOWILLVOIDYOURMANUFACTURER’S WARRANTY. DAMAGES RESULTING FROM MODIFICATIONS TO THIS FIXTURE AND/OR THE DISREGARD OF SAFETY INSTRUCTIONS AND GUIDELINES IN THIS MANUAL VOID THE MANUFACTURE’S WARRANTY AND ARE NOT SUBJECT TO ANY WARRANTY CLAIMS AND/OR REPAIRS. DO NOT PLUG FIXTURE INTO A DIMMER PACK! |
| Δ | NEVER OPEN THIS FIXTURE WHILE IN USE! UNPLUG POWER BEFORE SERVICING FIXTURE! NEVER TOUCH FIXTURE DURING OPERATION, AS IT MAY BE HOT! KEEP FLAMMABLE MATERIALS AWAY FROM FIXTURE! |
|  | NEVER LOOK DIRECTLY INTO THE LIGHT SOURCE! RETINA INJURY RISK - MAY INDUCE BLINDNESS! SENSITIVE PERSONS MAY SUFFER AN EPILEPTIC SHOCK! |
|  | INDOOR / DRY LOCATIONS USE ONLY! DO NOT EXPOSE FIXTURE TO RAIN AND MOISTURE! MINIMUM DISTANCE TO OBJECTS/SURFACES |
| Δ | MUST BE 39.4 FEET (12 METERS) MAXIMUM TEMP OF EXTERNAL SURFACE 194°F (90°C) MINIMUM DISTANCE OF INFLAMMABLE MATERIALS FROM THE SURFACE 0.7 FEET (0.2 METER) |

---12.0 m WARNING Minimum distance to lighted objects 12.0 meters. Maximum temperature of the external surface is 90°C Minimum distance of inflammable materials from the surface 0.2m.

## SAFETY  GUIDELINES

RISK  GROUP  3  -   RISK   OF   EXPOSURE  TO   ULTRAVIOLET					UV   RADIATION!   THE    FIXTURE   EMITS   A    HIGH-INTENSITY	WAVELENGTH  OF  ULTRAVIOLET  UV  LIGHT  FROM  THE   UV			COLOR FILTER. WEAR PROPER EYE AND SKIN PROTECTION.		AVOID PROLONGED PERIODS OF EXPOSURE TO THE UV COLOR					FILTER.  AVOID  WEARING  WHITE  CLOTHING  AND/OR  USING					UV  PAINTS  ON THE SKIN. AVOID  DIRECT  EYE AND/OR  SKIN					EXPOSURE AT DISTANCES LESS THAN 11 FEET (3.3M). DO NOT				OPERATE THE FIXTURE WITH DAMAGED/MISSING EXTERNAL			COVERS. DO NOT LOOK DIRECTLY INTO THE UV LIGHT AND/				OR VIEW UV LIGHT DIRECTLY WITH OPTICAL INSTRUMENTS	THAT  MAY  CONCENTRATE  THE   LIGHT/RADIATION   OUTPUT. INDIVIDUALS  SUFFERING  FROM  A  RANGE  OF  EYE  CONDITIONS,  SUNLIGHT  EXPOSURE DISORDERS,  OR  INDIVIDUALS  USING  PHOTOSENSITIVE  MEDICATION  MAY  EXPERIENCE DISCOMFORT IF EXPOSED TO THE ULTRAVIOLET UV LIGHT EMITTED FROM THE UV LED.

DO NOT TOUCH the fixture housing during operation. Turn OFF the power and allow approximately 15-minutes for the fixture to cool down before servicing.

DO NOT shake the fixture; avoid brute force when installing and/or operating the fixture.

DO NOT operate the fixture if the power cord is frayed, crimped, damaged, and/or if any of the power cord connectors are damaged and do not insert into the fixture securely with ease.

NEVER force a power cord connector into the fixture. If the power cord or any of its connectors are damaged, replace it immediately with a new one of a similar power rating.

DO NOT block any air ventilation slots. All fan and air inlets must remain clean and never blocked. Allow approximately 7.9” (20cm) between the fixture and other devices or a wall for proper cooling.

When installing the fixture in a suspended environment, always use mounting hardware that is no less than M10 x 25 mm, and always install the fixture with an appropriately rated safety cable.

Always disconnect the fixture from the main power source before performing any type of service and/ or cleaning procedure.

Only handle the power cord by the plug end; never pull out the plug by tugging the wire portion of the cord.

During the initial operation of this fixture, light smoke or a smell may emit from the interior of the fixture. This is a normal process and is caused by excess paint in the interior of the casing burning off from the heat associated with the lamp and will decrease gradually over time.

Consistent operational breaks will ensure the fixture functions properly for many years.

ONLY Use the original packaging and materials to transport the fixture for service.

## DISCHARGE  LAMP  WARNING

Q

Due to the  high  pressure  involved  in  the  construction  of  the  lamp,  it  MAY  EXPLODE  DURING PROLONGED EXTENSIVE USE. This risk increases with age; added care is encouraged when dealing with older lamps. Thus, the lamp must always be replaced at the end of its recommended duty cycle. Extreme caution should be used when operating this or any fixture fitted with a gas discharge lamp.

UV RADIATION NOTICE

Δ

THIS FIXTURE EMITS INTENSE UV RADIATION, WHICH IS HARMFUL TO THE EYES AND SKIN. THE INTENSE LUMINESCENCE OF THE LAMP CAN CAUSE SEVERE DAMAGE TO THE  RETINA.  NEVER OPERATE THIS FIXTURE WITH ANY OF THE PROTECTIVE  COVERS   REMOVED.  THESE  COVERS   HAVE   BEEN   SPECIALLY DESIGNED TO SHIELD AGAINST UV RADIATION.

This fixture is equipped with a DISCHARGE LAMP, which is highly susceptible to damage if mishandled. NEVER touch the lamp with your bare hands, as the oil from your hands will shorten the lamp’s life. NEVER move the fixture until the lamp has had ample time to cool. DISCHARGE LAMPS are NOT covered under warranty conditions. Avoid switching the fixture ON and OFF repeatedly in short intervals, as this will reduce lamp life and intensity. To achieve the intensity associated with discharge lamps, these lamps use gas sealed in a high-pressure environment to emit a brilliant output.

LAMP REPLACEMENT

Δ Δ

Δ

USE ONLY GENUINE ORIGINAL PHILIPS™ LAMPS. OTHER BRAND LAMPS WILL CAUSE DAMAGE AND VOID THE FIXTURE WARRANTY!

DISCONNECT THE MAIN POWER SUPPLY BEFORE REPLACING THE LAMP!

THE FIXTURE MUST COOL FOR 15 MINUTES BEFORE REPLACING THE LAMP! NEVER TOUCH THE LAMP WITH BARE HANDS; ALWAYS WEAR GLOVES!

OIL FROM HANDS WILL SHORTEN THE LIFE OF THE LAMP!

MAKE   SURE    ALL   COVERS/FIXTURES    ARE   REPLACED/SECURED   BEFORE OPERATING THE FIXTURE TO PREVENT ANY RISK AND/OR DAMAGE TO THE EYE RETINA FROM UV RADIATION EXPOSURE!

### DMX TRAITS

| CHANNEL | FUNCTION | DMX VALUE | EFFECT |
| --- | --- | --- | --- |
| 25CH |  |  |  |
| 1 | Cyan | 000 - 255 | Cyan 0 -100% |
| 2 | Magenta | 000 - 255 | Magenta 0 - 100% |
| 3 | Yellow | 000 - 255 | Yellow 0 - 100% |
| 4 | CTO | 000 - 255 | CTO 0 - 100% |
| 5 | Colour wheel | 000 - 005 | White color |
|  |  | 006 011 | White + color 1 |
|  |  | 012 017 | Color 1 |
|  |  | 018 - 023 | Color 1 + color 2 |
|  |  | 024 029 | Color 2 |
|  |  | 030 035 | Color 2 + color 3 |
|  |  | 036 - 041 | Color 3 |
|  |  | 042 047 | Color 3 + color 4 |
|  |  | 048 053 | Color 4 |
|  |  | 054 - 059 | Color 4 + Color 5 |
|  |  | 060 065 | Color 5 |
|  |  | 066 071 | Color 5 + color 6 |
|  |  | 072 - 077 | Color 6 |
|  |  | 078 083 | Color 6 + color 7 |
|  |  | 084 089 | Color 7 |
|  |  | 090 095 | Color 7+CTO |
|  |  | 096 174 | Clockwise rotation fast to slow |
|  |  | 175 176 | Stop |
|  |  | 177 255 | Counter clockwise rotation slow to fast |
| 6 | Strobe | 000 009 | Open |
|  |  | 010 199 | Pulse Effect slow to fast |
|  |  | 200 249 | Random Strobe |
|  |  | 250 255 | open |
| 7 | Dimmer | 000 - 255 | Dimmer 0 - 100% |
| 8 | DimmerFine | 000 - 255 | Dimmer Fine 0 - 100% |
| 9 | Static Gobo | 000 - 003 | Open |
|  |  | 004 - 007 | Gobo1 |
|  |  | 008 - 011 | Gobo2 |
|  |  | 012 - 015 | Gobo3 |
|  |  | 016 - 019 | Gobo4 |
|  |  | 020 - 023 | Gobo5 |
|  |  | 024 - 027 | Gobo6 |
|  |  | 028 - 031 | Gobo7 |
|  |  | 032 - 035 | Gobo8 |
|  |  | 036 - 039 | Gobo9 |
|  |  | 040 - 043 | Gobo10 |
|  |  | 044 - 047 | Gobo11 |
|  |  | 048 - 117 | Clockwise rotation fast to slow |
|  |  | 118 - 119 | Stop |
|  |  | 120 - 189 | Counter clockwise rotation slow to fast |
|  |  | 190 - 195 | Gobo1 Shaking Slow to Fast |
|  |  | 196 - 201 | Gobo2 Shaking Slow to Fast |
|  |  | 202 - 207 | Gobo3 Shaking Slow to Fast |
|  |  | 208 - 213 | Gobo4 Shaking Slow to Fast |
|  |  | 214 - 219 | Gobo5 Shaking Slow to Fast |
|  |  | 220 - 225 | Gobo6 Shaking Slow to Fast |
|  |  | 226 - 231 | Gobo7 Shaking Slow to Fast |
|  |  | 232 - 237 | Gobo8 Shaking Slow to Fast |
|  |  | 238 - 243 | Gobo9 Shaking Slow to Fast |
|  |  | 244 - 249 | Gobo10 Shaking Slow to Fast |
|  |  | 250 - 255 | Gobo11 Shaking Slow to Fast |

| 10 11 | Rotation Gobo Rotating Gobo Wheel | 000 - 007 | Open |
| --- | --- | --- | --- |
|  |  | 008 - 015 | Gobo1 |
|  |  | 016 - 023 | Gobo2 |
|  |  | 024 - 031 | Gobo3 |
|  |  | 032 - 039 | Gobo4 |
|  |  | 040 - 047 | Gobo5 |
|  |  | 048 - 055 | Gobo6 |
|  |  | 056 - 071 | Gobo7 |
|  |  | 072 - 113 | Continuous CCW rotation linear fast to slow |
|  |  | 114 - 117 | Stop rotation |
|  |  | 118 - 159 | Continuous CW rotationlinear slow to fast |
|  |  | 160 - 171 | Gobo1 Shaking Slow to Fast |
|  |  | 172 - 183 | Gobo2 Shaking Slow to Fast |
|  |  | 184 - 195 | Gobo3 Shaking Slow to Fast |
|  |  | 196 - 207 | Gobo4 Shaking Slow to Fast |
|  |  | 208 - 219 | Gobo5 Shaking Slow to Fast |
|  |  | 220 - 231 | Gobo6 Shaking Slow to Fast |
|  |  | 232 - 255 000 127 | Gobo7 Shaking Slow to Fast Gobo wheel position |
|  |  | 128 190 | Continuous CCW rotation linear fast to slow |
|  |  | 191 192 | Stop rotation |
|  |  | 193 - 255 | Continuous CW rotationlinear slow to fast |
| 12 | GoboRot Fine | 000 - 255 | Rotating Gobo Wheel Fine |
| 13 | Prism 1 | 000 127 | Prism out |
|  |  | 128 255 | Prism in |
| 14 | Prism1 Rotation | 000 127 | Prism position |
|  |  | 128 190 | Counter-Clockwise Rotation fast to slow |
|  |  | 191 192 | stop |
|  |  | 193 - 255 | Clockwise Rotation slow to fast |
| 15 | Prism2 | 000 127 | Prism out |
|  |  | 128 255 | Prism in |
| 16 | Prism2 Rotation | 000 127 | Prism position |
|  |  | 128 190 | Counter-Clockwise Rotation fast to slow |
|  |  | 191 192 | stop |
|  |  | 193 - 255 | Clockwise Rotation slow to fast |
| 17 | Frost | 000 255 | Frost Linear |
| 18 | Zoom | 000 - 255 | Zoom 0-100% |
| 19 | Focus | 000 - 255 | Focus 0-100% |
| 20 | Focus Fine | 000 - 255 | Focus Fine 0-100% |
| 21 | Pan | 000 - 255 | Pan 0-100% |
| 22 | PanFine | 000 - 255 | Pan fine 0-100% |
| 23 | Tilt | 000 - 255 | Tilt 0-100% |
| 24 | Tilt Fine | 000 - 255 | Tilt Fine 0-100% |
| 25 | Reset | 026 076 | Head reset |
|  |  | 077 - 127 | Pan and tilt reset |
|  |  | 128 - 255 | Whole light reset |