
LED Outdoor Wall Washer Manual

1. Graphic part：

E

[image: image1.jpg]QO OO




             

Function keys
Plus keys

Save button
Enter
Digital display window
A     B    C    D


2．Instructions: Operating instructions: Press the function key A to cycle out ten different function effects, the first two digits of the digital tube represent the current function (refer to the function table). The last two digits represent the address code or speed parameter where the function is located. Press B or C key to modify its parameter value. Press D to confirm.
LED display window function comparison table, (all the functions are selected and then press the D key to confirm)

	1
	A001
	Channel address code, (001—512)

B, C key plus and minus address code value

	2
	8CH
	Channel selection, (8CH/4CH)

B, C key plus and minus the number of channels

	3
	CC99
	Colorful jumps, (01-99)

B, C keys to modify the jump speed

	4
	 CP99
	Colorful pulse change, gradual change, (01-99)

B and C keys to modify the pulse change and gradual speed

	5
	DE99
	Colorful combination changes, (01-99)

B, C key to modify the speed of combination change

	6
	dEdF
	Colorful voice control changes

	7
	R255
	Red color selection, (000—255)

B and C keys to modify color selection

	8
	G255
	Green color selection, (000—255)

B and C keys to modify color selection

	9
	 B255
	Blue color selection, (000—255)

B and C keys to modify color selection

	10
	 W255
	White color selection, (000—255)

B and C keys to modify color selection


4. LED background display window function comparison table, (in DMXA001 state often press A key to enter, press D key to exit)

	1
	R255
	Red current, (020—255)

B and C keys to modify color selection

	2
	G255
	Green current, (020—255)

B and C keys to modify color selection

	3
	B255
	Blue current, (020—255)

B and C keys to modify color selection

	4
	W255
	White current, (020—255)

B and C keys to modify color selection

	5
	E001
	Console dimming speed 0-255

B and C keys to add and subtract values

	6
	C-55
	Temperature value setting (40-70)

B, C key repair temperature value


5、DMX8 Channel Features（8CH）

	Channel
	Function
	Description

	1
	Total dimming
	R, G, B, W total dimming, linear dimming, from dark to bright

	2
	Total strobe
	R, G, B, W total strobe, from slow to fast

	3
	Function selection
	0-5 empty

6-100 jumps;

101-150: Gradient;

151-200: Pulse change;

201-255: Voice control

	4
	Speed
	Function speed, from slow to fast

	5
	R dimming
	R dimming, linear dimming, from dark to bright

	6
	G dimming
	G dimming, linear dimming, from dark to bright

	7
	B dimming
	B dimming, linear dimming, from dark to bright

	8
	W dimming
	W dimming, linear dimming, from dark to bright


DMX4 Channel Features（4CH）

	Channel
	Function
	Description

	1
	R dimming
	R dimming, linear dimming, from dark to bright

	2
	G dimming
	G dimming, linear dimming, from dark to bright

	3
	B dimming
	B dimming, linear dimming, from dark to bright

	4
	W dimming
	W dimming, linear dimming, from dark to bright


7．Features function：

The operation is very simple, humanized design, dimming without flicker, jitter, etc. It can be adapted to occasions with strict lighting requirements such as photography, photography, and TV stations.
