[image: image1.jpg]


PRISM KING BEAM MOVE HEAD LIGHT

FOUR  PRISMS  

           USER MANUAL
[image: image16.png]



CONTENTS
1

Chapter 1  Installation and attention

1

1.1  Maintenance

1

1.2  Statement

1

1.3  Safety Precaution

1

1.4  Product Instruction

2

1.5  Cable connection（DMX）

2

1.6  Rigging (Optional)

4

Chapter 2  Panel operation

4

2.1  Brief

4

2.2  Operation

4

2.2.1 Operate light with touch or KEY

4

2.2.2 Parameter value setting

4

2.2.3 Boolean parameter setting

5

2.2.4 Sub Menu（Parameter）

5

2.3  Operation and parameter instruction

6

2.3.1 ADDR--> Address: Set DMX Address

6

2.3.2 MODE--> WorkMode: Set Light work mode

7

2.3.3 DISP-->DISPLAY: Set display

7

2.3.4 TEST--> TestMode

8

2.3.5 ADVA-->Advanced: Set light run parameter

9

2.3.6 STAT-->Status: View status

10

Chapter 3  Channel description

10

3.1  Channel table

。

3.2  Channel Detail
11
12。

3.2.1 COLOR Detail

12。

3.2.2 GOBO Detail



Chapter 1  Installation and attention 

1.1  Maintenance

To reduce the risk of electrical shock or fire, do not expose this unit to rain or moisture.
Intermittently using will extend this item’s service life.

Please clear the fan ,fan net , and optical lens in order to keep good work state.
Do not use the alcohol or any other organic solvent to wipe the shell.
1.2  Statement

The product has perfect performance and integrity packing. All users should be strictly complying with the warning and operating instructions as stated. Or we aren’t in charge of any result by misusing. Any damage resulting by misuse is not within the Company’s warranty. Any fault or problem caused by neglecting the manual is also not in the charge of dealers.

   Note: All information is subject to change without prior notice.

1.3  Safety Precaution
In order to guarantee the product’s life, please don’t put it in the damp places or even the environment over 60degress.

Always mount this unit in safe and stable matter.

Install or dismantle should operate by professional engineer.
Using lamp，the change rate of power voltage should be within±10%，If the voltage is too high，it will shorten the light’s life; If it’s not enough, will influence the effect.
Please restart it 20 minutes later after turning off light , until full-cooling. Frequent switching will reduce the life span of lamps and bulbs; intermittent using will improve the life of bulbs and lamps.
In order to make sure the product is used well, please read the Manual carefully.
1.4  Product Instruction

lamp:  MSD  (life:2200 hours Color temperature: 8000K)
Channel mode:16 DMX512 Channel
Pan scan：540°(16bit) Electric correction
Tilt scan：270° (16bit) Electric correction
Amazing dot matix, four tact switch, 180° turning show
Color wheel: one color wheel, 14 kinds of color chips in one color wheel
Gobo: 14 gobos
Effect Wheel:  Rotation 48 prism+8 prism, effect move , frost
0-100% mechanical dimming, mechanical dimming and free dimming available.
strobe macro control available.   
Lens optical system achanical fouce .beam angle 0～4°
Over heat protection
Power Input: 100-240V，50/60Hz
Power Dissipation：350W 
IP level :IP20
1.5  Cable connection（DMX）

Use a cable conforming to specifications EIA RS-485: 2-pole twisted, shielded, 120Ohm characteristic impedance, 22-24 AWG, low capacity. Do not use microphone cable or other cable with characteristics differing from those specified. The end connections must be made using XLR type 3 or 5-pin male/female connectors. A terminating plug must be inserted into the last projector with a resistance of 120Ohm (minimum 1/4 W) between terminals 2 and 3.
IMPORTANT: The wires must not make contact with each other or with the metal casing of the connectors. The casing itself must be connected to the shield braid and to pin 1 of the connectors.[image: image2.jpg]® DMX5120
5PINe

Sy e
Y + s

o

t

DMX5120

o
DMX512¢:
3PN

| S l@ | screens
soucr





Figure 1  DMX Cable connection

1.6  Rigging (Optional)

This equipment can be positioned and fixed by clamp in every direction of the stage. Locking system makes it easy to fasten to the bracket. 

Attention! Two clamps is needed to fix the equipment. Every clamp is locked by fastener of 1/4 kind. Fastener can only be locked clockwise.

Attention! Fasten a safety string to the additional hole of side aluminum piece. The secondary accessory can not hang on the delivery handle. Nip the equipment on bracket.

Check if rigging clamp (not including the one inside) damaged or not? If stand ten times weight as the equipment. Make sure the architecture can stand ten times weight as all the equipments, clamps, wirings and other additional fixtures.
Screws for clamping must be fixed firmly. Take one M12 screw (Grade 8.8 or higher) to clamp bracket, and then screw the nuts.
Level the two hanging points at the bottom of clamp. Insert fastener to the bottom, lock the two levers by 1/4 rotating clockwise; then install another clamp.
Install on safety string which stands at least ten times weight as equipment. Terminal of the accessory is designed for clamps.
Make sure pan/tilt lock unlocked or not. Keep the distance more than 1M from equipment to flammable material or lighting source.
[image: image3.png]


[image: image4.png]



Figure 2  Installation 
Chapter 2  Panel operation

2.1  Brief

The light panel diagram show as Figure 3, Left area is TFT Displayer, support touch, and right area is KEY, both of touch and KEY can operate light and setting.

Display & operation just like ‘Android operation system’, touch the item will set or modify setting. 

Note: Prevent damage the touch or TFT displayer, Can not use sharp objects chick displayer. 

[image: image5.jpg]18CHA[01:01:37 | NOR

ADDR | | scene

woe 001

pisp | 0101

| ADVA





Figure 3  Panel diagram
2.2  Operation

2.2.1 Operate light with touch or KEY

The left area is TFT Displayer and touch, chick item or value with finger will to complete operation of set light setting(parameters) or view light state.

The area on the right hand side is 4 KEY, As auxiliary input interface, if  disable touch function,, the KEYr can been choose to set the parameter.

2.2.2 Parameter value setting

When the selected item is value need to been modified, the dialog shown in Figure 4 will popup.

[image: image6.jpg]But no save

w008

Apply Value to system

—>

<«—Adjust Value quick

:) Adjust Value

Save value & quit




Figure 4  Dialog of value setting

Modify value：Can quickly modify value via pull the slider to the desired position, or click  the button of ‘up’ or ‘down’ whit finger on the right side to set the exact desired value, another way is roll encoder on the right hand side of panel.

Apply value：When Value had been modified, Then press the bottom of ‘apply’ in the left corner to apply to the light, but hav’t saved；

Save Value：Any time, click on the lower right corner of the "OK" button, the setting will been saved into internal memory.

2.2.3 Boolean parameter setting

when the selected parameters is a Boolean value (such as ON or OFF), can directly modify setting by chick corresponding item, the setting will been saved right now. 
When the parameter is a key item, chick corresponding item, a dialog shown in Figure 5 will been popup ask for the confirm. Chick ‘sure’ to confirm.

[image: image7.png]



Figure 5  Dialog of confirm

2.2.4 Sub Menu（Parameter）

Chick item of main menu, enter corresponding sub menu, shown in Figure 6, total 6 sub menu, includes class of parameter and status:

ADDRESS：Set light DMX address.

WORKMOD：Set light work mode, master or slave mode when in auto run mode.

DISPLAY： Set display parameter, eg. select language.

TEST：Used for test light, modify DMX channel data to test function， the corresponding function  of reference channel function table.

ADVANCE：Set light running parameter.

STATUS：view light current status.

[image: image8.png]R out oeet | [roseess s Enelisn
Workdode  Auto Worklode Scroan saver o
Biggiag] Sound otr! Display | Soroon rotation _OFF
et VS chcore o | L Toun Emble o
L switen o Toush gectity
e
statun statun
Escope Eacape
hadress | oan o0 AN tnsert or | [asdress worcuose ow
Norkdoda| TILT o0 TILT Inset o | | Wrokboda Adaress oo
Display PO o0 ey 0 | Lolaplay Version 11 160
Tentiose | O o0 PN Offeat oo | | e Eleose oo oa
w0 o0 TILT Offaet o0 Tatol 00000 o
Advwced 50 o0 Lo hen Fonero | A
Stts st o0 atn old o | [ | o q—
Escape | sTROBE o0 Factoy senvins Escape





Figure 6  Parameter menu

2.3  Operation and parameter instruction

Via following operation, enter sub menu(parameter menu) shown in Figure 6
In main menu, chick 1/6 function button into corresponding parameter menu.

In sub menu(page), chick main item on the left side of displayer, can shift to corresponding sub menu(page) quickly.

2.3.1 ADDR--> Address: Set DMX Address

Click and select the "ADDR", can enter the page of DMX address setting, range from 1 to 512, the address code shouldn’t is not greater than (512- channels quantity), otherwise the light will not been controlled. Following is the operation: 

Enter the page of DMX address, as shown in Figure 7, click the blank area in right side of display will pop-up diglog as in Fig. 4, modify value, then click ‘ENTER’ to confirm and save DMX address code.

[image: image9.png]Address
WorkMode
Display
TestMode

Advanced

Status

Escape





Figure 7  page of DMX Address

2.3.2 MODE--> WorkMode: Set Light work mode

Enter the page of ‘WorkMode’ as shown in Figure 8 and modify setting. Can set light work mode, control lamp and DMX channel mode.

[image: image10.png]Address | DMX Ctrl

WorkMode | Auto Run
Sound Ctrl

M/S choose

Display
TestMode

Light Switch

Advanced ¢ el Qty

Status

Escape




Figure 8  page of work mode
   ◆ DMX Ctrl:     Choose to set  DMX Mode,

   ◆ Auto Run:     Choose to set  Auto Mode, 

   ◆ Sound Ctrl:    Choose to set  Sound Mode,

   ◆ M/S Choose:Available just in ‘AUTO RUN’ or ‘SOUND Ctrl’ mode.
      ON--> Master. (Data will be send to other slave lamp immediately.)

     OFF--> Slaver.(NOT send data to other lamp via DMX Cable).(Default)  

   ◆ Light Switch:  

      ON--> Turn on the light, 

     OFF--> Turn off the light.

   ◆ Channel Qty: Light support 2 DMX Channel mode: sample or extend。
     Simple --> 16CH.(Default)  

     Expand--> 20CH(or null).
2.3.3 DISP-->DISPLAY: Set display

Light support 2 language, rotation display，Enter page as shown in Figure9 to set parameter following:

[image: image11.png]Address | EE English

WorkMode | Screen saver Mode3
Display | Screen rotation OFF
| Touch Enable ON
TestMode
Touch Rectify
Advanced

Status

Escape





Figure9   page of display
   ◆ Language:  English / 中文.
   ◆ Screen Saver: when panel is idle(these is no operation in 10 second), displayer will enter saver status.
      OFF--> No screen saver.

  Mode1--> Power-saving mode, turn off the display.

  Mode2--> Displays the current address.

  Mode3--> Displays the icon and the current working mode.(Default)

   ◆ Screen Rotion: To turning display.
      ON--> Normal display.(Default)

  OFF--> 180° turning display.
   ◆ Touch enable：Disable or enable touch function,.

      ON--> Enable touch function.(Default)

  OFF--> Dosable touch function.

   ◆ Touch adjust：Adjust touch function. Normally, not enter this item.

2.3.4 TEST--> TestMode

Enter the page as shown in Figure 10, Light will into test mode, in this mode, the light does not receive the data for DMX controller.:

[image: image12.png]Address | PAN 000
Workllode | TILT 000

Display | FOCUS 000
TostMode | 0Lk 000

60BO 000
Advanced | ppy 000

Status ‘ FROST 000
Escape | STROBE 000




Figure 10  page of Test
   ◆ PAN:      range for 0 to 255;

   ◆ TILT:      range for 0 to 255;

   ◆ FOCUS:    range for 0 to 255;

   ◆ COLOR:    range for 0 to 255;

   ◆ GOBO:     range for 0 to 255;

   ◆ PRISM:     range for 0 to 255;

   ◆ FROST:     range for 0 to 255;；

   ◆ STROBE:   range for 0 to 255;

2.3.5 ADVA-->Advanced: Set light run parameter

Enter the page as shown in Figure 10, set the parameter of light:

[image: image13.png]Address
WorkMode
Display
TestMode

Advanced

Status

Escape

PAN Insert
TILT Inset
P/T Rectify
PAN Offset
TILT Offset

| Lamp when

Data hold
Factory Setting

OFF

OFF

N

010

010
Power ON

OFF





Figure 11  page of run parameter

   ◆ Pan Invert:  Reverse PAN move
      OFF--> Pan Normal move.(Default)
       ON--> Reverse PAN move.
   ◆ Tilt Invert:  Reverse TILT move
      OFF--> Tilt Normal move.(Default)

       ON--> Reverse Tilt move.
   ◆ P/T Rectify: Disable or enable position rectify function.
      OFF--> Disable P/T rectify

       ON--> Enable P/T rectify-(Default)
   ◆ Pan Offset:     Set PAN original position. Default: 10
   ◆ Tilt Offset:      Set TILT original position. Default: 10
   ◆ Lamp when:    

      PowerON--> Turn on the lamp when power on.(Default)
      RstDone--> Turn on the lamp after reset.
      Manual--> Manually turn on the lamp.
   ◆ Data hold:     

      OFF--> When no DMX signal,return to middle position.(Default)
       ON--> When no DMX signal,stop in the final position.
   ◆ Factory Setting:  Restore all parameter to factory setting.

2.3.6 STAT-->Status: View status 

Enter the page as shown in Figure 12:

[image: image14.png]Address
WrokMode
Display
TestMode

Advanced

Status

Escape

Work Mode DMX ...

Address 001

Version B5R.1.1 16n
Elapse 000H 04M
Tatol 00000H 04M





Figure 12  page of status

    ◆ Work Mode:  Show the current working mode.
    ◆ Address:     Show the current address.

    ◆ Version:      Show the version of the lamp.

    ◆ Elapse:      Working hours after turn on.
    ◆ Tatol:        Cumulative hours of operation
[image: image15.png]



Chapter 3  Channel description

3.1  18 Channel table

Table 1  Channel brief

	18CH
	22CH
	Name
	number
	describe

	CH1
	CH1
	Color
	0-4
	White

	
	
	
	5-9
	White+Color1

	
	
	
	10-14
	Color1

	
	
	
	15-19
	Color1+Color2

	
	
	
	20-24
	Color2

	
	
	
	25-29
	Color2+Color3

	
	
	
	30-34
	Color3

	
	
	
	35-39
	Color3+Color4

	
	
	
	40-44
	Color4

	
	
	
	45-49
	Color4+Color5

	
	
	
	50-54
	Color5

	
	
	
	55-59
	Color5+Color6

	
	
	
	60-64
	Color6

	
	
	
	65-69
	Color6+Color7

	
	
	
	70-74
	Color7

	
	
	
	75-79
	Color7+Color8

	
	
	
	80-84
	Color8

	
	
	
	85-89
	Color8+Color9

	
	
	
	90-94
	Color9

	
	
	
	95-99
	Color9+Color10

	
	
	
	100-104
	Color10

	
	
	
	105-109
	Color10~Color11

	
	
	
	110-114
	Color11

	
	
	
	115-119
	Color11+Color12

	
	
	
	120-124
	Color12

	
	
	
	125-129
	Color12+Color13

	
	
	
	130-134
	Color13

	
	
	
	135-139
	Color13+Color14

	
	
	
	140-144
	Color14

	
	
	
	145-149
	Color14+White

	
	
	
	150-200
	Forwards rainbow from fast to slow

	
	
	
	201-255
	Backwards rainbow from slow to fast

	CH2
	CH2
	Strobe
	0-3
	Close

	
	
	
	4-103
	Fadeout slow to fast

	
	
	
	104-107
	Open

	
	
	
	108-207
	Free Strobe slow to fast

	
	
	
	208-212
	Open

	
	
	
	213-251
	Pulse Strobe slow to fast

	
	
	
	252-255
	Open

	CH3
	CH3
	Dimmer
	0~255
	Dimmer intensity from 0% to 100%

	CH4
	CH4
	Gobo
	0-4
	White

	
	
	
	5-9
	Gobo1

	
	
	
	10-14
	Gobo2

	
	
	
	15-19
	Gobo3

	
	
	
	20-24
	Gobo4

	
	
	
	25-29
	Gobo5

	
	
	
	30-34
	Gobo6

	
	
	
	35-39
	Gobo7

	
	
	
	40-44
	Gobo8

	
	
	
	45-49
	Gobo9

	
	
	
	50-54
	Gobo10

	
	
	
	55-59
	Gobo11

	
	
	
	60-64
	Gobo12

	
	
	
	65-69
	Gobo13

	
	
	
	70-125
	Forwards rainbow from fast to slow

	
	
	
	126-130
	Stop

	
	
	
	131-190
	Backwards rainbow from slow to fast

	
	
	
	191-195
	Gobo1 Shake，Slow to fast

	
	
	
	196-200
	Gobo2 Shake，Slow to fast

	
	
	
	201-205
	Gobo3 Shake，Slow to fast

	
	
	
	206-210
	Gobo4 Shake，Slow to fast

	
	
	
	216-220
	Gobo6 Shake，Slow to fast

	
	
	
	221-225
	Gobo7 Shake，Slow to fast

	
	
	
	226-230
	Gobo8 Shake，Slow to fast

	
	
	
	231-235
	Gobo9 Shake，Slow to fast

	
	
	
	236-240
	Gobo10 Shake，Slow to fast

	
	
	
	241-245
	Gobo11 Shake，Slow to fast

	
	
	
	246-250
	Gobo12 Shake，Slow to fast

	
	
	
	251-255
	Gobo13 Shake，Slow to fast

	CH5
	CH5
	Prism
	0-31
	Empty

	
	
	
	32-63
	Insert Prism 1

	
	
	
	64-95
	Insert Prism 2

	
	
	
	96-127
	Insert Prism 3

	
	
	
	128-159
	Insert Prism 4

	
	
	
	160-191
	Insert Prism 1+2

	
	
	
	192-223
	Insert Prism 1+3

	
	
	
	224-255
	Insert Prism 1+4

	CH6
	CH6
	Prism 1.Rot
	0-127
	Prism position

	
	
	
	128-190
	Forwards rainbow from fast to slow

	
	
	
	191-192
	Stop

	
	
	
	193-255
	Backwards rainbow from slow to fast

	CH7
	CH7
	Prism wheel.Rot
	0-127
	Prism position

	
	
	
	128-190
	Forwards rainbow from fast to slow

	
	
	
	191-192
	Stop

	
	
	
	193-255
	Backwards rainbow from slow to fast

	CH8
	CH8
	Colorful/ Frost
	0-127
	empty

	
	
	
	128-190
	Frost

	
	
	
	191-255
	colorful

	CH9
	CH9
	Zoom
	0~255
	Zoom Large to small

	CH10
	CH10
	PAN
	0~255
	Pan movement by 540

	CH11
	CH11
	PAN fine
	0~255
	Tilt movement by 270

	CH12
	CH12
	TILT
	0~255
	Fine control of pan movement

	CH13
	CH13
	TILT fine
	0~255
	Fine control of tilt movement

	CH14
	CH14
	Macro function
	0~255
	Macro function

	CH15
	CH15
	Reset
	0-25
	empty

	
	
	
	26-76
	Small motor reset

	
	
	
	77-127
	Big motor reset

	
	
	
	240-255
	Reset all(over 5 seconds)

	CH16
	CH6
	Lamp control
	0-9
	empty

	
	
	
	10-100
	Lamp off（Over 3 seconds）

	
	
	
	101-255
	Lamp on（Over 3 seconds）

	CH17
	CH17
	Led Strip
	0-255
	Effect Show 

	CH18
	CH18
	Led Strip Speed
	0-255
	Effect Show Speed

	
	CH19
	NO Effect
	
	Self-defined

	
	CH20
	NO Effect
	
	Self-defined

	
	CH21
	NO Effect
	
	Self-defined

	
	CH22
	NO Effect
	
	Self-defined


Please read over this manual before operation the light





When <Data hold> set <ON>,click to clear DMX data, and make the lamp return to themiddle position.





Click to reset.








PAGE  

